To enable an in-depth investigation of the dominant mechanisms governing the system and to elucidate the consumption and production of oxygen atoms within the system, method of source identification of O element was introduced into the analysis. For this purpose, the kinetic model of H2/(NO/N2O/N2) combustion was rewritten. In the oxygen-labeled combustion kinetic model, both oxygen-containing species and their elementary reactions were supplemented with corresponding labeled forms (O). The thermodynamic and transport properties of each O-labeled species were kept consistent with those of their unlabeled counterparts. For elementary reactions involving only one oxygen-containing species in both reactants and products, the rate constants of the O*-labeled reaction were maintained identically to those of the original reaction. For reactions involving two or more oxygen-containing species, special treatment was required. Detailed rewriting methods are provided in Table 3 and 4.

Table 3	Composition of nitrogen elements before and after source identification, considering the symmetry and asymmetry of the substances being distinguished.

| Before differentiation | After differentiation | Before differentiation | After differentiation |
| --- | --- | --- | --- |
| O | O* | HNO | HNO* |
| OH | O*H | HON | HO*N |
| H2O | H2O* | HONO | HONO*, HO*NO, HO*NO* |
| HO2 | HOO*, HO*2 | H2NO | H2NO* |
| H2O2 | H2OO*, H2O*2 | HNOH | HNO*H |
| O2 | OO*, O*2 | HNO2 | HNOO*, HNO*2 |
| NO | NO* | NO3 | NOOO*, NOO*O*, NO*3 |
| N2O | N2O* | HNO3 | HNOOO*, HNOO*O*, HNO*3 |
| NO2 | NOO*, NO*2 | NH2OH | NH2O*H |

Table 4	Composition of nitrogen elements before and after source identification, considering the symmetry and asymmetry of the substances being distinguished (N2O in case (ii)), the units for A, Ea are (cm3/mol)n-1and cal/mol.

| Case | Before source identification | A | b | Ea | After source identification | A | b | Ea |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (i-1) | N+OH=NO+H N2O(+M)=N2+O(+M) NO+H(+M)=HNO(+M) | 3.360E+13 .9900E+11 4.48E+19 | 0 0 -1.32 | 385 57901 740 | N+O*H=NO*+H N2O*(+M)=N2+O*(+M) NO*+H(+M)=HNO*(+M) | 3.360E+13 .9900E+11 4.48E+19 | 0 0 -1.32 | 385 57901 740 |
| (i-2) | HNO+O=NO+OH 考虑化学键 | 3.61E+13 | 0 | 0 | HNO*+O=NO*+OH HNO+O*=NO+O*H HNO*+O*=NO*+O*H | 3.61E+13 | 0 | 0 |
| (ii-1) | (1) OH+OH=O+H2O duplicate OH+OH=O+H2O duplicate | 1.35E+07 -2.697E+10 | 1.689 0.567 | -1166.8 0.0 | (2) O*H+OH => O*+H2O DUPLICATE O*H+OH => O*+H2O DUPLICATE O*+H2O => O*H+OH !REV! (3) O*H+OH => O+H2O* DUPLICATE O*H+OH => O+H2O* DUPLICATE O+H2O* => O*H+OH !REV! (4) O*H+O*H => O*+H2O* DUPLICATE O*H+O*H => O*+H2O* DUPLICATE O*+H2O* => O*H+O*H !REV! | (2) 1.35E+07 -2.697E+10 2.382E+08 (3) 1.35E+07 -2.697E+10 2.382E+08 (4) 1.35E+07 -2.697E+10 2.382E+08 | (2) 1.689 0.567 1.616 (3) 1.689 0.567 1.616 (4) 1.689 0.567 1.616 | (2) -1166.8 0.0 14484.4 (3) -1166.8 0.0 14484.4 (4) -1166.8 0.0 14484.4 |
| (ii-2) | O+O+M=O2+M | 6.165E+15 | -0.50 | 0.00 | O+O*+M = OO*+M !kf*=2×kf REV/ /!kr*=kr O*+O*+M = O*2+M | 1.233E+16 2.145E+17 6.165E+15 | -0.50 -0.555 -0.50 | 0.00 118676.8 0.00 |
| (iii-1) | N2O+O=NO+NO | 9.200E+13 | 0.00 | 2.767E+04 | N2O*+O=NO*+NO N2O+O*=NO*+NO N2O*+O*=NO*+NO* | 9.200E+13 9.200E+13 9.200E+13 | 0.00 0.00 0.00 | 2.767E+04 2.767E+04 2.767E+04 |
| (iii-2) | H2O2(+M)=OH+OH(+M) | 2.00E+12 | 0.90 | 4.8749E+04 | H2OO*(+M)=O*H+OH(+M) REV/ / !kr*=2×kr H2O*2(+M)=O*H+O*H(+M) | 2.00E+12 5.720E+07 2.00E+12 | 0.90 1.73 0.90 | 4.8749E+04 -1832.6 4.8749E+04 |
| (iv-1) | N2O+O=N2+O2 | 3.700E+12 | 0.00 | 1.593E+04 | N2O*+O=N2+OO* REV/ / !kr*=0.5×kr N2O+O*=N2+OO* REV/ / !kr*=0.5×kr N2O*+O*=N2+O*2 | 3.700E+12 8.390E+08 3.700E+12 8.390E+08 3.700E+12 | 0.00 0.86 0.00 0.86 0.00 | 1.593E+04 94704.7 1.593E+04 94704.7 1.593E+04 |
| (iv-2) | O2+H=O+OH | 1.0400E+14 | 0.00 | 15286.0 | OO*+H=O*+OH!kf*=0.5×kf REV/ / !kr*=kr OO*+H=O+O*H!kf*=0.5×kf REV/ / !kr*=kr O*2+H=O*+O*H | 0.5200E+14 6.802E+11 0.5200E+14 6.802E+11 1.0400E+14 | 0.00 0.290 0.00 0.290 0.00 | 15286.0 -1302.4 15286.0 -1302.4 15286.0 |
| (v) | NO2+HO2=HONO+O2 考虑化学键结构 HONO的氧都来自NO2 | 1.910E+00 | 3.32 | 3.044E+03 | NOO*+HO2=HONO*+O2 NO2+HOO*=HONO+OO* NOO*+HOO*=HONO*+OO* NO2+HO*2=HONO+O*2 NO*2+HO2=HO*NO*+O2 NOO*+HO*2=HONO*+O*2 NO*2+HOO*=HO*NO*+OO* NO*2+HO*2=HO*NO*+O*2 | 1.910E+00 1.910E+00 1.910E+00 1.910E+00 1.910E+00 1.910E+00 1.910E+00 1.910E+00 | 3.32 3.32 3.32 3.32 3.32 3.32 3.32 3.32 | 3.044E+03 3.044E+03 3.044E+03 3.044E+03 3.044E+03 3.044E+03 3.044E+03 3.044E+03 |

Note: The probability of reaction pathway rewriting should not be determined solely based on mathematical statistical probability; attention must also be paid to the bond dissociation patterns involved, such as those occurring in species like HONO.